## Determining POGO® Actuator Program Position

POGO® System operation begins by loading a program for the part and the index devices onto the system controller. A CAD/CAM system (i.e. CATIA, Unigraphics, etc.) is used to develop POGO® System programs by overlaying the part over a representation of the POGO® system, and then using the available functions to extract pocket position and Z-axis height information.  This information is then post-processed into the text based format that the POGO® System requires.

The POGO® actuator move distance is measured from the zero reference plane to the contact surface of the end effector (when it is horizontal).  Rather than trying to determine exactly where the contact surface meets the part a simple process is used to determine the required program point for each POGO® actuator.  The initial step is to develop a representation of the POGO® system on a CAD/CAM system using simple elements such as lines, points, and planes, or create solid models of the system, keeping the geometry as simple as possible.  Any indexing fixtures used for positioning the part on the POGO® system will also need to be modeled.  The process for determining the POGO® actuator program position is as follows:

- Using a CAD/CAM system position the part over the representation of the POGO® system.  It may be necessary to move the part around within the work envelope in order to optimize the number of actuators holding the part and the part overhang.

- Use CAD/CAM system functions to offset the part the same distance as the end effector distance.  The pivot-to-contact distance in the adjacent diagram is .850”.

Note:  The pivot-to-contact distance varies depending on the end effector model being used.  Reference the end effector specifications in the POGO® Flexible Tooling System Maintenance Manual to obtain the proper value.

- Use CAD/CAM system functions to offset the part surface the same distance as the pivot-to-contact distance, in the direction of the zero reference plane.

- Use CAD/CAM system functions to intersect the centerline of the actuators with the part surface and extract the distance from this intersection point to the zero reference plane.  The results of this step provide the distance from the zero reference plane to the center of the end effector pivot.

- Add the pivot-to-contact distance to the value in Step 4 for the POGO® actuator program height.  At this distance the end effector will pivot to contact the part properly.

Note:  Typically Step 5 is accomplished using a post processor, which takes the POGO® actuator locations, X/Y axis position, and the Z-axis heights, adds the pivot-to-contact distance and generates a POGO® system program file with the proper command syntax.

# POGO System Programs

Each line of a program may contain one command.  Commands are delimited by the carriage return and/or linefeed character(s).  Spaces at the beginning of a line and completely blank lines are ignored. To improve readability, multiple spaces may be used to separate and align column as in the example below.  Comments, delimited by the ‘;’ character, may appear on lines by themselves or at the end of a line containing a command.

## POGO System Programs - Stand Alone

A stand-alone program consists of one or more program definition blocks contained within a single disk file that may be added directly to the “Available Programs” list.  The following example contains all of the commands and formatting that most POGO® system programs require.

Example:

DEFINE_PRG    Part46B7    V2.7

MOVE    S01R01C01    14.005

MOVE    S01R01C06     8.123  ; a comment

MOVE    S01R01C07     8.240

MOVE    S01R02C01     9.653

MOVE    S01R01C03    10.449

END_DEFINE_PRG

The program definition starts with the “DEFINE_PRG” command.  This includes the program name “Part46B7” and the version “V2.7”.  Program Name and Version may be anything you want, but cannot contain embedded spaces.

Actuator commands follow the “DEFINE_PRG” command statement.  This example contains MOVE commands referencing 5 actuators.  The structure of the MOVE command as shown in the second line above contains the command, “MOVE”, the Pocket Address, “S01R01C01”, and the Goal Position, “14.005”.  The Pocket Address is composed of Section, Row, and Column coordinates that uniquely identify the pocket.  The Goal Position must be specified in the same “Units of Measure”, Millimeters or Inches, that the POGO® System will be using when the program is run.

Reference section “Error: Reference source not found” for information on additional actuator commands.